FOR CHATGPT PROMPT

There will be 5 sources of data:

- rolling 24hour rainfall at upper catchment, Jackson creek and deep creek area

- river level upstream measured at Keilor gauge

- river level downstream measured at Maribyrnong and Footscray gauge

- major drainage inflow flow meter; assuming 80L/sec capacity pipe

- major drainage outlet flow meter; assuming 80L/sec capacity pipe

- telecom reflective level estimate for downstream measured at Maribyrnong and Footscray gauge

The rules to determine whether it is normal, watch, high risk or severe is as below:

| Source | Watch | High risk | Severe |
| --- | --- | --- | --- |
| Rolling 24 hour rainfall at upper catchment, Jacksons Creek and Deep Creek | 25 to 45 mm | 45 to 70 mm | 70+ mm |
| River level upstream, Keilor gauge | 2.5 to 3.5 m | 3.5 to 4.0 m | 4.0+ m |
| River level downstream, Maribyrnong and Footscray gauge | 1.2 to 1.7 m | 1.7 to 2.9 m | 2.9+ m |
| Major drain inflow meter | 48 L/sec – 64 L/sec | 64 L/sec – 76 L/sec | 76 L/sec |
| Telecom reflective level estimate | 1.2 to 1.7 m | 1.7 to 2.9 m | 2.9+ m |

The rule is if 2/5 data shows watch, it is watch; 2/5 data show high risk; it is high risk. 2/5 data show severe, it is severe.

Another feature the website should have other than identifying flood risk is identifying blocked drainage. The rule to determine that is as below:

- if (inflow rate – outflow rate)  >  20L/sec    there is a small blockage

- if (inflow rate – outflow rate)  >  40L/sec    there is a big blockage

- if (inflow rate – outflow rate)  >  60L/sec    there is a severe blockage

We want to be able to feed the data into the website and see real time change in flood risk analysis. Ie real time change from watch to high risk. Ideally, we can visually see how all 6 sources of data changes over time. For example a real time plotting graph for each data source.

For Chrisio

There are 10 sheets of data that I gave you, test all 10 sheets. The correct answer is labelled.